SCHAAL DRILLING, CO.

46881 Hwy 24    BURLINGTON,  CO.  80807

719-346-8032

 FIELD REPORT

SURFACE CASING, DRILLING AND CEMENTING
DRILLING DATE    7-16-11
WELL NAME & LEASE #  Roll SWD 843-18-21
MOVE IN, RIG UP, DIG PITS ETC.  DATE        7-15-11

TIME    10:00  AM  PM
SPUD TIME  5:00  AM PM     SHALE DEPTH  190

TIME      9:15  AM  PM
HOLE DIA.  12     FROM 0 TO  475 +12   487  TD
            
 TIME   12:30   AM  PM
CIRCULATE, TOOH, MEASURED  464.40  FT CASING  476.40  KB FT SET DEPTH

       7     JOINTS    8 5/8    0D #/FT     17   20   23   24
PUMP   200   SAC CEMENT   1107   GALLONS DISP. CIRC.   14  BBL TO PIT

PLUG DOWN TIME   3:30  AM  PM
  DATE   7-16-11
CEMENT LEFT IN CSG.  50
TYPE : PORTLAND CEMENT I/II ASTM C 150

CENTRALIZER   3


CEMENT COST $   
CENTRALIZER SET@  1-2  5-6  9-10
ELEVATION  3702






+12KB             3714










PIPE TALLY



1.  42.25

11.42.20
2.  42.25

12.
3.  42.20

13.
4.  42.25

14.
5.  42.20                      15.




6.  42.25

16.



7.  42.10

17.
8.  42.25

18.
9.  42.25
                                    Total

10.42.20

     464.40

