SCHAAL DRILLING, CO.

46881 Hwy 24    BURLINGTON,  CO.  80807

719-346-8032

 FIELD REPORT

SURFACE CASING, DRILLING AND CEMENTING
DRILLING DATE  7-07-11
WELL NAME & LEASE #   D&S 43-22
MOVE IN, RIG UP, DIG PITS ETC.  DATE        7-06-11

TIME  11:45    AM  PM
SPUD TIME  7:15  AM PM  SHALE DEPTH   320
            
TIME  10:00    AM  PM
HOLE DIA.  9 7/8     FROM 0 TO  396  +12  408  TD
            TIME  11:00    AM  PM
CIRCULATE, TOOH, MEASURED  391.70  FT CASING  403.70  KB FT SET DEPTH

       9     JOINTS    7    0D #/FT     17   20   23   16.04
PUMP  98   SAC CEMENT     630   GALLONS DISP. CIRC.   8   BBL TO PIT

PLUG DOWN TIME  1:00  AM  PM
  DATE   7-07-11
CEMENT LEFT IN CSG. 30  

TYPE : PORTLAND CEMENT I/II ASTM C 150

CENTRALIZER   1


CEMENT COST $   1607.13
CENTRALIZER SET@  1-2
ELEVATION  3891




+12KB             3903







PIPE TALLY



1.  43.65

11. 
2.  44.05

12. 
3.  44.00

13. 
4.  43.60

14.
5.  43.90                     15.




6.  43.95

16.



7.  40.15

17.
8.  44.10

18.
9.  44.30
                                    Total

10. 

              391.70

