H2S Safety Plan – Low Risk Conditions

Drilling and Completing Gothic Shale Wells

(1)  H2S monitoring equipment will be on site while drilling and completing.  Submit plans for H2S monitoring via sundry notice.

(2)  A map shall be posted at the well site showing all facilities within a half (1/2) mile radius where the public is likely to be present (applicable to both drilling and fracturing operations).  Also, post names, addresses and as available, phone numbers of the public who normally live, work or congregate within a half (1/2) mile of the well site. 

(3)  72 hours prior both to spud & fracturing operations, notify COGCC Inspector Steve Labowskie at 970-259-0945 or steve.labowskie@state.co.us

Provide the following information: Operator Name, Well Name and Number, API #, Spud Date, Contact Name, Contact Phone #, Email Address

(4)  72 hours prior to spud & fracturing operations, notify county dispatch & Local Governmental Designee of potential for H2S 

(5)  Have H2S scavenger on site at time of drilling out surface casing shoe 

(6)  If H2S is encountered, collect a sample representative of the reservoir conditions and have a lab determine the H2S concentration in ppm as soon as practicable.  Measure or estimate absolute open flow potential of the well.  Report the results and calculations to the COGCC verbally first and ASAP then by email and Sundry Notice.

(7)  If H2S is encountered, shut in well and cease operations

(8)  If H2S is encountered, notify county dispatch, COGCC, & local governmental designee of H2S presence

(8)  If H2S is encountered and the radius of exposure equals or exceeds 50 feet, submit for COGCC approval PUBLIC H2S PROTECTION PLAN prior to resuming operations

(9)  Submit H2S Operations Plan for approval & Implement prior to resuming operations

*BLM Onshore Order No. 6 – Hydrogen Sulfide Operations (see COGCC Instructions & Guidance Document)

MEW (1-29-10)
MEW(8-11-10) Revised
MEW (12-9-10) Revised

MEW (3-29-12)