
[image: image1.png]1103. ABANDONMENT

Each pipeline abandoned in place shall be disconnected from all sources and supplies of natural
gas and petroleum, purged of liquid hydrocarbons, depleted to atmospheric pressure, and cut off
three (3) feet below ground surface, or the depth of the pipeline, whichever is less and sealed at
the ends. This requirement shall also apply to compressor or gas plant feeder pipelines upon
decommissioning or closure of a portion or all of a compressor station or gas plant. Notice of such
abandonment shall be filed with the Commission and with the local governmental designee or
local government jurisdiction.



Memorandum

To: 

Well File 
From:

Mark Weems, P.E. – District Engineer, SW Colorado
Date:

September 17, 2012
Subject:
Landowner Request
EPNG
Butler Pool Unit #44-28-1

SENW 28 34N 12W

05-067-05538

Current Conditions

Above ground dry hole marker in place; Cement well cellar walls still in place; gathering pipeline valve in place found west of the PA marker; gathering pipeline riser locate about ¼ west of the PA marker.
Current Landowners

Tom and Elaine Hartnet

970-259-7726

Landowners would like to have all equipment removed to facilitate farming operations.

Anadarko

Gathering pipeline operator and subsequent owner of properties previously owned and operated by Western Gas Resources (WGR) and El Paso Natural Gas (EPNG).
As per phone conversation today, with Kent McEvers, contract consultant and previous gas plant superintendent for WGR/Anadarko, the following points were made:

Proper pipeline abandonment - Rule 1103 page 1103 effective date April 1, 2009  

(See below)
[image: image3.wmf]
P&A marker removal:

1.  Only the COGCC registered & bonded operator of the well may remove the PA marker.

2.  Any change in operator requires the submittal and approval of a form 10 – Change of Operator.

3.  The operator must submit and receive prior COGCC approval of their plans and procedure to cut off a PA marker.  This is to be done via a form 4 – Sundry Notice.  Schematics or drawings depicting both the current as built P&A marker construction and the proposed final configuration of the cut off PA marker must also accompany the sundry notice. 

Cc: Kent McEvers (505-598-5601 ext. 15523 office; kent.mcevers@anadarko.com )
� EMBED Imaging.Document  ���








2

[image: image2.wmf]_1133606233.bin

