LEASE DESCRIPTION:  Schlachter 743-6-31

    
                          Black Raven Energy

LOCATION: 414’ FNL & 2408’ FEL, LOT 2 Section 6-T7N-R43W

ELEV: 3679' GR

COUNTY: Phillips STATE: Colorado API #: 05-095-06332-00 

             FIELD: WILDCAT

11/29/11: MIRU SCHAAL DRILLING CO. PREVIOUS 24 HRS - SPUD SURFACE@7:30 A.M. ON 11/28/11. 9 7/8” HOLE DRILLED 0’ TO 478’ TD@5:30 P.M. CIRC. TOOH.  RAN 12 JTS OF 7”,17# CSG MEAS 461.20’. SET@473.20’. CMT W/100 SKS OF TYPE 1 NEAT CMT. CIRC 5 BBL TO PIT. PLUG DOWN@7:15 .PM. ON 11/28/11. RDMO.

SHALE @ 270’

EDC: $5073

12/02/11:  MIRU ADT #144. NIPPLE UP B.O.P. ASSEMBLE BHA. MIX MUD. DRILL OUT CMT PLUG. SPUD @ 11:00 A.M. ON 12/01/11. DRILL FROM 473’ TO 2658’. TD @ 2658’@ 7:00 PM. CIRC. POOH. LAY DOWN BHA. RIG UP PSI. RAN TRIPLE COMBO LOGS. LOGGER TD @ 2690’. TOP 2396’. BTTM 2434’. AV. 1.5, MAX 2, 34% PEROSITY, 34’ ZONE. RDMO PSI. TIH TO P&A, RIG UP CMTRS. 1ST PLUG 35 SKS 2350’ TO 2150’, 2ND PLUG 35 SKS 570’ TO 400’, 3RD PLUG 10 SKS 50’ TO 4’. RELEASE RIG @ 8:30 P.M. RDMO RIG # 144.

SURVEYS:

500’ -     1.3

1000’ -  .31

1500’ – .23

2000’ – 1.28

2600’ –   .10

ECD:
$56,588

