Laramie Energy II L.L.C.
Casing and Cement Record

	Well Name       
	Hawxhurst 24-09D
	Date
	4/28/ 2011

	Field / Area
	Buzzard 
	Report By
	Kellyy Claussen


	#
	
	
	
	
	
	
	Overall
	Effective
	Setting
	Depth

	Jts.
	 Size
	  Wt.
	 Gr.
	Cplg.
	Rge.
	Cond.
	Length
	Length
	Set From
	Depth To

	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	    
	-4.00
	KB

	35
	9-5/8”
	36#
	J-55
	STC
	3
	1
	
	1543.26
	KB
	1539.26

	1
	Float
	Collar
	
	
	
	
	
	1.00
	1539.26
	1540.26

	1
	9- 5/8” 
	36#
	J-55
	STC
	3
	1
	
	26.44
	1540.26
	1566.44

	1
	Guide
	shoe
	
	
	
	1
	
	1.00
	1566.44
	1567.44

	36
	Total Jts.
	
	
	
	
	Total
	1571.17


	Casing Crew / Run Time
	Weatherford TRS  Rig up and ran casing 3.5 hrs.  



	Centralizers
	Middle of  shoe joint, top of 2nd , 3rd , 5th &  then every 3rd collar to 40’, 

  Total of 13 centralizers.

	Stage 1
	

	Circulation Time / Pres. 
	Circulate on bottom for 1 hr @ 280#.

	Pre-Flushes
	20 bbls of Fresh water, 


	Cement  Type / Volume
	 120 sx  Lead of  Halliburton Versacem System cement, Mixed @ 12.3#/gal. Followed w/ 200 sx Tail of Halliburton  Swiftcem system mixed @ 14.2/gal.


	Displacement Info.
	121.5 bbls of water

 Bumped Plug to 1300 psi .  Float held.  C.I.P. @ 1200 hrs.  4/28/11


	Stage 1 Comments
	Returns were full with 5 bbl of cement returns.  

Cement level remained  at surface.   

	
	

	1” top job
	None required

	Circulation Time / Pres.
	

	Pre-Flushes
	

	Cement  Type / Volume
	

	Displacement Info.
	

	Stage 2 Comments
	


Mud Properties before Cementing

	9.4
	54
	16
	19
	19-51
	10.8
	8.3
	7.6
	1050
	4/28/11

	WT.
	VIS
	PV
	YP
	GELS
	WL
	PH
	SOLIDS
	CHLORIDE
	LAST REPORT


Other Remarks

	    

	

	 

	

	


