APC-Charlotte #2-3

Re-Entry Operational Plan

SE NE 3-17S-45W

Kiowa County, CO

Drilling Plan

- Dig out 4-1/2” casing.  Weld on collar.  Install BOP.

- RU pulling unit with power swivel & pump. Set circulating tank.

- Fill tank with produced lease water.

- GIH with bit, drill collars and tubing

- Drill out 10 sack cement plug at surface, 10 sack cement plug at 500’ and 10 sack cement plug at 2700’.

- Drill out CIBP at 2750’.

- Drill out 20 sack cement plug at 4350’.

- Run tubing and bit down to top of cut tubing at 4375’.  Circulate hole clean.

- TOH laying down collars.

Completion Plan

- Install wellhead.

- Set CIBP above tubing at 4370’ with 2 sacks cement on top.

- Swab casing down to 3500’.

- Perforate Lansing from 4272’-4278’.

- Swab test – treat if necessary.

- Run tubing and rods for production.

- RD & MOL.