Plan of Workover Operation on Cased Hole Well

Hanavan 15-1 (API 05-017-07749) 

Formerly Hanavan #1 

SWSE Sec 15, T13S R44W, 6th PM, Cheyenne Cty, CO

1) Excavate Existing Casing Stubs


a) 8 5/8” 24# set to 422’ and cemented to surface

b) 5 ½” 15.5# 
set to 5510’ and cement 5510-5100’, 2583-1100’, 450’-shoe

2) Weld on new casing for BOPE

3) MIRU Codell Workover Rig w/ drilling package

4) Drill out cement plugs at surface, 120-229’, 450’ and pressure test casing to 300 psi


a) if no pass, find holes and squeeze leaks with cement till pass pressure test

 
b) if pass pressure test drill out CIBP @5330’

5) Perform cement squeeze of perfs at 5346-5362’

6) Drill out squeeze and pressure test casing


a) if no pass, repeat squeeze and drill out


b) if pass, perf 5385-5410 in the Spergen Formation

7) Test Spergen perfs


a) if commercial complete well with tubing, rods, and pump jack

b) if uncommercial plug well as previously done or as consistent with COGCC requirements, whichever is more stringent

