# Wellbore Diagram        API Well No:	05-087-07205-00-

Owner:	RITCHIE EXPLORATION, INC.	Well Name:	MERINO STATE #3

County:	MORGAN	Field:	MESSEX	Pool:

Coordinates: X	1320 FEL	;Y	330 FSL	Sec:	36	Twp:	6N	Rng:  55W

Note:Changes to the drawing do not effect the database

PRIOR PA

# Bore Diameters (in)

Cement Top: 0' mthd:U

Cement Top:0’ mthd:U

8 5/8” Surf 129’

Probable Casing Leak @ 1025’

CMT Top @ 4548’   _____________

Perts. 5,447-5,458'

5 1/2" 1ST	5,733'

Total Vertical     Depth 5, 734'      =========

Pkr with on/off tool – no tubing @ 5005’

J- Sand Perfs. 5,083’-5,086’  _______

PBTD @ 5109’

4 ½” 1ST  5,148’

Total Vertical Depth 5,150’