Andy’s Mesa Gas Processing Plant COGCC Field Inspection Notes (5-6-13)

NGL Storage Area:

Southeast edge of Location (Fac. ID 269202)

Concrete supports noted of Google Earth image (6-10-13) are no longer on site.

Compressor Building:

2 - AST in secondary containment ~250 gal. each

Approximately 3” of fluid w/ oil in containment

1 - 55 gal. unlabeled drum, appears to be full of unknown contents

Spoil pile north of compressor

Approximately 16’ by 24’ by 2’ in height tarped area within secondary containment berm

Lube oil leaks from equipment

Large bird’s nest inside compressor building, other evidences of wildlife residing within structure

Shallow trough surrounding equipment providing containment for lube oil spills

Debris and unused equipment surrounding and within compressor building

Processing Building Site:

Large debris pile including pipe line materials

~25’ by 60’ tarped spoils pile similar to the one near compressor building, evidence of secondary containment berm.

~500 gal. septic tank inside steel stock tank

~300 gal. tank labeled thermguard 50

Note: This equipment appears to have been staged as part of deconstruction.

~25’ by 50’ stock pile of oily soil, tarped and bermed.

2 unlabeled fiberglass vaults.

400 bbl AST:

Inside lined earthen berm

Valve leak on north side of tank

Labeled Condensate/Produced Water

Thief hatch was open and venting to the atmosphere

“Flare Stack” Area:

Appears to have earthen berms surrounding discolored black soil.

Lean-To Shed:

Unused equipment including well head parts, valves, empty 55 gal. drums, and trash, as well as other debris

Filters within secondary containment, oily residue within structure

Drum Storage Area:

5 - stock tanks with oil and other fluids, some on pallets others on bare ground. Evidence of multiple spills onto unprotected ground. No spill protection or secondary containment. Pallets and plastic sheeting within tanks.

~200 gal. chemical tote, half full of fluid

19 - 55 gal. drums, some marked with oily water/diesel water. No spill protection

Oil tank with secondary containment

2 - steel tanks, one labeled Methanol and the other labeled Geo-815

Air compressor Generator Building:

Used oil filters outside of building, some staining present on ground.

Oily stains on concrete slab, 2 - 5 gal. gas cans and other debris inside structure

2 - 5 gal. buckets on west side of building with unknown fluids

Storage Garage:

Unused equipment storage

Possible underground storage tank, unlabeled

“Slop Tank” Area:

Unused equipment, including 2 separator houses and two steel pipe equipment protectors to the east

Debris and trash including lube grease tubes leaking onto the ground