WELL PLUGGING PROCEDURE

Curton Capital Corporation

4925 Greenville Ave. #800

Dallas TX 75206

(469) 726-2222

Well: United 5A-s-5

API# 103-40671

Reference form #6- 2361863

Well has been plugged

BLM representative was on location

Procedure:

Back filled well with surface material (shale) to 50 feet below surface. Cemented well from 50 feet below surface to 3 feet below surface. Cut casing off 3 feet below surface. Welded marker plate (identification plate) to casing which included operator name, legal description, date plugged with weep hole. Then buried well to surface, the well site has not been reclaimed at this time.

Chris Curton

Company representative

720-308-0306 cell