Met COGCC inspector Jason Gomez in the Vista Ridge neighborhood in Erie to verify odor was present at approximately 10:00 a.m.  Was able to smell the odor intermittently.  Smelled like diesel fuel.  Due to the transient nature of the odor a canister sample was not taken.  Using a PPBRae 3000 handheld PID sampling device I was able to confirm the presence of diesel fuel in the air.  From 10:15 to approximately 10:30, I walked along  Hickory Drive with the device and detected levels ranging between 40 ppb and 180 ppb while the odor was present.  Levels did return to zero when no smell was present.  I drove to the playground area and also walked along the walking path directly between the rig and the neighborhood.  From approximately 10:45 to 11:00 a.m.I was able to again detect the presence of diesel fuel in the air intermittently.  The levels along the walking path ranged from 20 to 45 ppb when the odor was present.  Both the areas I walked with the detector are marked by the red lines in the map below.  There was a slight, variable breeze at the time of sampling, generally from a northeasterly direction (according to wind data obtained from the Erie Municipal Airport).  The wind direction is indicated by the yellow arrow.  At approximately 11:00 a.m. I met with the APCD inspector Rebecca Wilson.  She indicated she had received several complaints from the neighborhood that morning regarding the smell.  Left the area at 11:20 a.m.

Please let me know if you need any additional information from me in regards to this event.  Thanks!

​