Marick 1A								P.O.&G. Operating, LLC

API #05-121-08461							5847 San Felipe, Suite 3200

Ground level = 4799’							Houston, TX 77057

Perfs = 4318’-4321’							Phone 713-244-9986

Casing = 5 ½” 							FAX 713-244-0650

PBTD = 4342’

ESP= 3,986’
____________________________________________________________________________________________________________

PROCESS REVIEW

Marick 1A was shut in since 9/1/2018 and PO&G Operating is currently working to remediate the oil carryover problem that was documented on August 1, 2018.

Timeline:

9/1/2018: Marick 1A was SI due to evaporation pits reaching max capacity.

9/5/2018: Robert young filed COGCC field inspection report from August 1, 2018 documenting oil carry over into the Marick 1A skim pit.

9/7/2018: As the pits were dropping we noticed that there was a bathtub ring of hydrocarbons along the dried banks that would contaminate the water. Roustabout crew removed hydrocarbon ring to prevent future contaminations.

Phase 1:  Chemicals (Increased treating rate in conjunction with new Demulsifier)
	Start Date: ~September 15, 2018

  - 2X/Month batch treatments down backside of well with demulsifier 2491

  - Confirm that oil/water levels in FWKO/HT are appropriately set

  - Once well reaches steady state conduct Millipore analysis on both vessels water legs to determine if any oil is being carried over to water tanks

  - If we continue to face oil carrying over into the pit we will move into phase 2 that incorporates facility upsizing & reducing rate

Phase 2: Facility Upsizing / Reducing Rate

  - Facility Upsizing

    - Install larger/secondary FWKO to increase retention time

  - Decrease rate of ESP

    - Install VFD to slow down ESP

    - Install smaller ESP

    - Change artificial lift method

Attachments:

- Marick 1A Facility Overview