NESG 14-24 Plug & Abandon

API#: 05-123-24977

SURFACE: 1,208' FSL 900' FWL

SW SW 24 3N 68W

WELD COUNTY, COLORADO

| ENGR: | Clark Martin | ALT. ENGR:* | Jay Stockton |
| --- | --- | --- | --- |
| OFFICE: | 970-336-3872 | OFFICE: | 432-685-5951 |
| CELL: | 817-229-5160 | OFFICE | 713-366-5246 |
| HOME: |  | CELL: | 432-967-5644 |

RECOMMENDED PROCEDURE:

Pre-Rig Up Work:

- Move in and rig up slickline unit. Pull plunger, springs and standing valves as per normal procedure.

- Prep well for pulling unit.

- Record and report tubing, casing and bradenhead pressures.

Rig Procedure:

Note: Notify district COGCC office 24 hours before commencing work and before all cementing operations.

- Move in and rig up P&A rig with associated equipment. Rig up pump equipment to wellhead. Pressure test pump and lines to +/- 3,000 psi.

- Record shut-in tubing, casing and bradenhead pressures.

- Kill tubing & casing with brine or fresh water with biocide, pumping +/- 80 bbls down casing and then +/- 30 bbls down tubing to flush all hydrocarbons out of wellbore. Maintain injection rate of minimum 6 – 8 BPM during flush operations. All kill & flush fluids to be biocide treated.

- Nipple down wellhead.

- Nipple up BOP equipment. Pressure test BOP equipment in accordance with current practices.

- Rig up rig floor and remaining equipment.

- Pull out of hole laying down +/- 187 joints of 2 3/8” tubing. Stand back remaining 50 joints in derrick.

- Move in and rig up wireline. Run in hole with 4.5” 11.6# gauge ring to ~7,500’. Pull out of hole with gauge ring.

- Run in hole and set Niobrara/Codell 4.5” CIBP at +/- 7,200’.

- Run in hole and dump bail 2 sx of cement on top of CIBP set at 7,200’. Pull out of hole with wireline and dump bailer.

- Load well with Fresh Water containing biocide and test 4.5” production casing to 500 psi for 15 minutes.

Note: A successful pressure test must have less than a 10% loss and a minimum of 15 minutes of stabilized pressure.

- Run in hole and set Sussex 4.5” CIBP at 4,032’.

- Run in hole and dump bail 2 sx of cement on top of CIBP set at 4,032’. Pull out of hole with wireline and dump bailer.

- Connect 2 – 2” lines onto the Bradenhead, and lay 1 – 2” line to the waste tank.

- Run in hole with 3 1/8” Select Fire Perforating Guns.  Perforate squeeze holes              at +/- 2,000’ with 3 1/8” gun, 3 SPF, 120° phasing, 12 shots.

- Pull Wireline up to +/- 1,500’ and perforate squeeze holes with 3 1/8” gun, 3 SPF, 120° phasing, 12 shots. Pull out of hole with wireline and spent gun & rig down Wireline.

- Run in hole with 4.5” Tension Packer on 50 joints 2 3/8” Tubing & set Packer ~1,560’.

- Open Tubing & Tubing Annulus.  Circulate down 2 3/8” Tubing at 6 BPM with Fresh Water with biocide out perfs at ~2,000’ & taking returns up tubing annulus, using ~150 bbls Fresh Water.

- Switch to 12.0 ppg Mud.  Pump down 2 3/8” Tubing at 6 BPM with 12.0 ppg Mud until return fluid weighs 12.0 ppg and is clean mud. Use ~100 bbls 12.0 ppg Mud.

- Unset 4.5” Tension Packer and pull out of the well.

RIH with 4.5” CICR on 50 joints Tubing and set at ~1,560’.  Place tubing in “Test Mode” and pressure test tubing to ~2,000 psi for 10 mins.			 									                                Note: Heat all cement mixing water to 70° F. All circulating rates need to be at 6 - 8 BPM to promote hole cleaning and residual gas removal.

Note:  On all AGM cement jobs, the Rig Crew must be ready to pull Tubing.  The Rig pump must be primed on mud & engine running ready to Reverse Circulate, and hoses switched for Reverse Circulation & when necessary return fluid out Bradenhead to the waste tank.

- Move in and rig up cement equipment. Test pump and lines to 3,000 psi for 10 mins.

- Mix and pump Class G AGM cement down Tubing at 6 BPM at ~1,560’.

- Pump 25 bbls Pre-Flush, 25 bbls Fresh water followed by 220 sx Class G AGM with 0.4% CJ522R Latex, 2% CJ110 Calcium Chloride and 4% CJ101 gypsum mixed at 15.8 ppg.  Pull mixing tub down as far as possible. Do not stop to wash up. Obtain dry & wet samples of cement.

- Go straight into displacement. Displace cement with ~4 bbls of 12 ppg mud.

- Lay down 12 joints Tubing (~378’), end of Tubing ~1,190’.

- Reverse circulate at 6 BPM using 12.0 ppg Mud until Tubing return fluids are clean.  Use ~40 bbls 12.0 ppg Mud.

- Pull all ~38 joints out of the well & remove Mechanical Setting tool.  Fill 4.5” Casing with 12.0 ppg Mud.

- RIH with ~38 joints Tubing at ~1,190’.

- Mix and pump Class G AGM cement down Tubing at 6 BPM at ~1,190’.

- Pump down 2 3/8” Tubing at 6 BPM 10 bbls Fresh Water followed by 35 sx Class G AGM with 0.4% CJ522R Latex, 3% CJ110 Calcium Chloride and 5% CJ101 gypsum mixed at 15.8 ppg.  Obtain dry & wet samples of cement.

- Pump ~2 bbls 12.0 ppg Mud displacement.

- Lay down 10 joints Tubing (316’).  End of Tubing ~651’.  Reverse Circulate at 6 BPM 12.0 ppg Mud until Return Fluids are clean 12.0 ppg Mud, using ~20 bbls Mud.

- Lay down all the Tubing.

- MIRU Wireline and RIH to punch ~3 – 4 holes in 4.5” Casing at ~600’.  Pull out of hole with wireline.  Rig down Wireline.

- If no pressure, ND BOPE and install B1 adapter.  Connect 2 – 2” flow back lines from the Bradenhead to the waste tank and 1 HP Cementing line to a valve on the 4.5” Casing.

- Open Bradenhead.  Open Casing.  Pump 12.0 ppg Mud down 4.5” Casing at 6 BPM taking returns out the Bradenhead for a Minimum ~80 bbls until return mud is clean and weighs 12.0 ppg.

Note: Heat all cement mixing water to 70° F. All circulating rates need to be at 6 - 8 BPM to promote hole cleaning and residual gas removal.

Note:  On all AGM cement jobs, the Rig Crew must be ready to pull Tubing.  The Rig pump must be primed on mud & engine running ready to Reverse Circulate, and hoses switched for Reverse Circulation & when necessary return fluid out Bradenhead to the waste tank.

- Test pump and lines to 3,000 psi for 10 mins.

- Mix and pump Class G AGM cement down Casing at 6 BPM at 600’.

- Pump 20 bbls Pre-Flush, 20 bbls Fresh water followed by 190 sx Class G AGM with 0.4% CJ522R Latex, 3% CJ110 Calcium Chloride and 5% CJ101 gypsum mixed at 15.8 ppg and clean Class G AGM cement circulates to the waste tank.  Obtain dry & wet samples of cement.

- Place 200 psi on well and shut-in.

- Rig down and move off pulling unit and related equipment.

- Instruct cementing and wireline contractors to e-mail copies of all job logs/job summaries to rscDJVendors@anadarko.com within 24 hours of completion of the job.

- Supervisor submit paper copies of all invoices, logs, and reports to VWP Engineering Specialist.

- Excavation crew to notify One Call to clear excavation area around wellhead and for flow lines.

- Excavate hole around surface casing enough to allow welder to cut casing a minimum 5’ below ground level.

- Welder cut casing minimum 5' below ground level.

- Spot weld on steel marker plate.  Marker should contain Well name, Well number, legal location (1/4 1/4 descriptor) and API number.

- Obtain GPS location data as per COGCC Rule 215 and send to rscDJVendors@anadarko.com.

- Properly abandon flow lines per Rule 1103.  File electronic Form 42 once abandonment is complete.

- Back fill hole with fill.  Clean location, and level.

- Submit Form 6 to COGCC ensuring to provide 'As performed' WBD identifying operations completed.

Well Capacity Factors:

2.375” Tubing	0.0039 bbls/ft

2.875” Tubing	0.0058 bbls/ft

4.5” Csg	0.0155 bbls/ft

2.375” x 4.5” Csg    0.0101 bbls/ft

4.5” x 8.625” 	      0.0481 bbls/ft

4.5” x 7.875” Hole   0.0406 bbls/ft

4.5” Csg x 8.625”   0.0636 bbls/ft

7.875” Hole	     0.0602 bbls/ft