Site: RODGERS #1 FLOWLINE

08/25/2022

Site Inspection Report

# Site Inspection Report

# Inspection Details

## Site/Location; ## Date; ## Time; ## Inspector. RODGERS #1 FLOWLINE; 08/25/2022; 03:55 PM; COGCC Representative. Comments. Work Activity Notes; Crews were continuing to repurpose the flowline at the west excavation. A soil sample was collected at the base of the spill excavation. Changes since previous inspection?; Yes; Drive by inspection; No. Actively being worked?; Yes. Fieldwork Oversite Performed?; Yes; Oversite Type; Soil sampling. Were Oversite Samples Collected?; Yes; Sample Details; Marcom and I split one base sample at the bottom of the spill excavation. The PID reading was 800+ ppm. The sample was run for Table 915-1 Organics.

Checklists

| Remediation Activity Checklist |  |
| --- | --- |
|  | Photo |
| Question: Open excavation(s)? Note: Approximate size(s) (Length x Width x Depth, ft) Response: Yes Notes: 4 open excavations. Two at the ends of the flowline and two spill locations. |  |
| Question: Open excavation(s)? Note: Approximate size(s) (Length x Width x Depth, ft) Response: Yes Notes: 4 open excavations. Two at the ends of the flowline and two spill locations. |  |
| Question: Open excavation(s)? Note: Approximate size(s) (Length x Width x Depth, ft) Response: Yes Notes: 4 open excavations. Two at the ends of the flowline and two spill locations. |  |
| Question: Open excavation(s)? Note: Approximate size(s) (Length x Width x Depth, ft) Response: Yes Notes: 4 open excavations. Two at the ends of the flowline and two spill locations. |  |
| Question: Stockpiled E&P waste and approximate volume (cubic feet)? Response: Yes Notes: Both stockpiles are clean. | No Photo |
| Question: Groundwater present within excavation? Note: Depth to water below grade surface (ft), free product/sheen Response: No Notes: | No Photo |
| Question: Monitoring well(s) present? Note: Number, approximate location(s), condition Response: No Notes: | No Photo |
| Question: Soil impacts remain in situ? Response: Yes Notes: | No Photo |

| Site Condition Checklist |  |
| --- | --- |
|  | Photo |
| Question: E&P waste signage present, legible, accurate and up to date (labelled per KPK CWMP)? Response: Yes Notes: |  |
| Question: Emergency signage in compliance with 605.g.? Note: Location of signage - Location entrance, Tank Battery, Wellhead(s). Response: No Notes: No signage | No Photo |
| Question: Ground surface appears clean (no evidence of new equipment leaks, oil leaks, spills, or releases)? Response: No Notes: |  |
| Question: Ground surface appears clean (no evidence of new equipment leaks, oil leaks, spills, or releases)? Response: No Notes: |  |
| Question: Ground surface appears clean (no evidence of new equipment leaks, oil leaks, spills, or releases)? Response: No Notes: |  |
| Question: Ground surface appears clean (no evidence of new equipment leaks, oil leaks, spills, or releases)? Response: No Notes: |  |
| Question: Open excavation properly fenced/covered (in accordance with Rule 913.b.(5)B.i.)? Response: Yes Notes: |  |
| Question: Open excavation properly fenced/covered (in accordance with Rule 913.b.(5)B.i.)? Response: Yes Notes: |  |
| Question: Open excavation properly fenced/covered (in accordance with Rule 913.b.(5)B.i.)? Response: Yes Notes: |  |
| Question: Open excavation properly fenced/covered (in accordance with Rule 913.b.(5)B.i.)? Response: Yes Notes: |  |
| Question: Open excavation properly fenced/covered (in accordance with Rule 913.b.(5)B.i.)? Response: Yes Notes: |  |
| Question: Is fencing adequate/appropriate for site and well(s) and maintained? Response: Yes Notes: |  |
| Question: Is fencing adequate/appropriate for site and well(s) and maintained? Response: Yes Notes: |  |
| Question: Is fencing adequate/appropriate for site and well(s) and maintained? Response: Yes Notes: |  |
| Question: Exposed buried/underground flowlines within excavation? Response: Yes Notes: |  |
| Question: Exposed buried/underground flowlines within excavation? Response: Yes Notes: |  |
| Question: Exposed buried/underground flowlines within excavation? Response: Yes Notes: |  |
| Question: If flowlines are exposed are they properly capped (bullnose plug, etc.)? Response: Yes Notes: | No Photo |
| Question: Site is protected from vehicular traffic (properly fenced, gated or barricaded)? Response: Yes Notes: |  |
| Question: Site is protected from vehicular traffic (properly fenced, gated or barricaded)? Response: Yes Notes: |  |
| Question: Site is protected from vehicular traffic (properly fenced, gated or barricaded)? Response: Yes Notes: |  |
| Question: Is earthen berm secondary containment present and in good condition (compacted, complete, no evidence of erosion, animal burrowing, etc.)? Response: Yes Notes: |  |
| Question: Is earthen berm secondary containment present and in good condition (compacted, complete, no evidence of erosion, animal burrowing, etc.)? Response: Yes Notes: |  |
| Question: Is work being conducted in a minimal footprint? Note: Any property/crop damage Response: Yes Notes: | No Photo |
| Question: Is location adjacent to any Potential Receptors? Note: Livestock, surface water, occupied structure, adjacent land use, etc. Response: Yes Notes: Neighborhood |  |
| Question: Stockpiled waste stored and handled in compliance with Rule 913.b.(5).B.iv.? Response: Yes Notes: | No Photo |

| Stormwater Checklist |  |
| --- | --- |
|  | Photo |
| Question: Evidence of pollutants being discharged off-site? Response: No Notes: | No Photo |
| Question: All stormwater erosion control measures and BMPs are present and in good/working condition? Response: Yes Notes: | No Photo |
| Question: Site free of unused equipment, weeds, debris/trash? Response: Yes Notes: | No Photo |

GENERAl site Photos

| General Site - 1 | General Site - 2 |
| --- | --- |
| General Site - 3 | General Site - 4 |
| General Site - 5 | General Site - 6 |
| General Site - 7 | General Site - 8 |
| General Site - 9 | General Site - 10 |
| General Site - 11 |  |